SECTION 16510

LIGHTING

PART 1		GENERAL

1.1	SUBMITTALS

A.	Shop Drawings:  Provide the following documentation of each lighting fixture (luminaries) as follows, but not limited to:

1.	Factory cut-sheets of each fixture, outlining all physical, electrical and photometric characteristics,

2.	Typical wiring diagram of each fixture, including emergency circuit components or any similar optional features,

3.	Lamp characteristics, including type, size, ANSI designation, etc.,

4.	Driver characteristics, including type, input watts, operation amps, ballast factor, etc.

1.2	QUALIFICATIONS

A.	Manufacturer:  Company specializing in manufacturing the Products specified in this section with minimum five (5) years documented experience.

1.3	REGULATORY REQUIREMENTS

A.	Conform to requirements of NFPA 70.

B.	Conform to requirements of NFPA 101.

C.	Products:  Listed and classified by Underwriters Laboratories, Inc. as suitable for the purpose specified and indicated.

# PART 2	PRODUCTS

2.1	FIXTURES

A.	Fixtures are scheduled and specified on drawings.

B.	Application:  All fixtures shall be rated for the environment in which they will be installed.  Fixtures in this location will be installed in a tropical, humid environment with standing water surrounding the area.

C.	Corrosion-Resistance: All fixtures shall be constructed to resist corrosion.  Housings shall be painted, corrosion-resistant aluminum; non-metallic fixture housings will be also considered, but must be submitted for approval.

# PART 3	EXECUTION

3.1	INSTALLATION

A.	Make wiring connections to branch circuit using building wire with insulation suitable for temperature conditions.

B.	Bond fixtures and metal accessories to branch circuit equipment grounding conductor,

3.2	ADJUSTING

A.	Aim and adjust floodlights as directed in the field.

3.3	CLEANING

A.	Clean electrical parts to remove conductive and deleterious materials.

B.	Remove dirt and debris from enclosures.

C.	Clean finishes and touch up damage.

3.4	DEMONSTRATION

A.	Demonstrate fixture operation for minimum of three (3) hours.

3.5	PROTECTION OF FINISHED WORK

A.	Repair luminaries that have failed at Substantial Completion to provide all luminaries operating as intended by the manufacturer.

B.	Replace any damaged components or entire fixture if necessary.

- END SECTION -