SECTION 03300

CAST-IN-PLACE CONCRETE

# GENERAL

## 1-01	SCOPE:

### The work of this Section consists of furnishing all labor, equipment, supplies, and materials necessary for the proper placement, curing, finishing, protection, and repair of the cast-in-place concrete required by the Contract Documents.

B.	Related work specified elsewhere:

#### SECTION 02220 Excavation and Backfill

#### SECTION 03100 Concrete Formwork and Accessories

#### SECTION 03200 Concrete Reinforcement

#### SECTION 03600 Grout

## 1-02	APPLICABLE PUBLICATIONS:

## A.	The following standard specifications shall apply to the Work of this Section:

### 1.	American Concrete Institute (ACI)

#### a.	ACI 117 - Standard Tolerance for Concrete Construction and Materials.

#### b.	ACI 301 - Structural Concrete for Buildings

#### c.	ACI 304.2R - Recommended Practice for Measuring, Mixing, Transporting, and Placing Concrete

#### d.	ACI 305 - Committee Report on Hot-Weather Concreting

#### e.	ACI 306 - Committee Report on Cold-Weather Concreting

#### f.	ACI 308 - Standard Specification for Curing Concrete

#### g.	ACI 309 - Consolidation of Concrete

#### h.	ACI 318 - Building Code Requirements for Reinforced Concrete

### 2.	American Society for Testing and Materials (ASTM):

#### a.	C 31 - Making and Curing Concrete compression and Flexure Test Specimens in the Field

#### b.	C 33 - Concrete Aggregates

#### c.	C 39 - Compressive Strength of Cylindrical Concrete Specimens

#### d.	C 94 - Ready-Mixed Concrete

#### e.	C 127 - Test Method for Specific Gravity and Absorption of Coarse Aggregate

#### f.	C 128 - Test Method for Specific Gravity and Absorption of Fine Aggregate

#### g.	C 136 - Method for Sieve Analysis of Fine and Coarse Aggregates

#### h.	C 143 - Test Method for Slump of Hydraulic Cement Concrete

#### i.	C 150 - Portland Cement

#### j.	C 156 - Test Method for Water Retention by Concrete Curing Materials

#### k.	C 157 - Test Method for Length Change of Hardened Hydraulic Cement Mortar and Concrete

#### l.	C 173 - Standard Test Method for Air Content of Freshly Mixed Concrete by the Volumetric Method

#### m.	C 231 - Standard Test Method for Air Content of Freshly Mixed Concrete by the Pressure Method

#### n.	C 260 - Specification for Air Entraining Admixtures for Concrete.

#### o.	C 309 - Liquid Membrane-Forming Compounds for Curing Concrete

#### p.	C 494 - Chemical Admixtures for Concrete

#### q.	C 566 - Test Method for Total Moisture Content of Aggregate by Drying

#### r.	C 618 - Standard Specification for Coal Fly Ash and Raw or Calcined Natural Pozzolan for Use as a Mineral Admixture in Concrete

#### s.	C 881 - Standard Specification for Epoxy-Resin-Base Bonding Systems for Concrete

#### t.	C 1077 - Practice for Laboratories Testing Concrete and Concrete Aggregates for use in Construction and Criteria for Laboratory Evaluation

#### u.	C 1240 - Standard Specification for Silica Fume for Use as a Mineral Admixture in Hydraulic-Cement Concrete, Mortar, and Grout

#### v.	D 1751 - Standard Specification for Preformed Expansion Joint Filler for Concrete Paving and Structural Construction (Nonextruding and Resilient Bituminous Types)

#### w.	D 2419 - Test Methods for Sand Equivalent Value of Soils and Fine Aggregate

#### x.	E 96 - Water Vapor Transmission of Materials

#### y.	E 1643 - Standard Practice for Installation of Water Vapor Retarders Used in Contact with Earth or Granular Fill Under Concrete Slabs

### 3.	Federal Specifications

#### a.	UU-B-790A - Building Paper, Vegetable Fiber (Kraft, Waterproofed, Water Repellant and Fire Resistant).

### 4.	International Bulding Code 1B6

## 1-03	DEFINITIONS:

### A.	Structural Concrete:  Concrete to be used in all cases except where indicated otherwise in the Contract Documents.

### B.	Pea Gravel Concrete:  Concrete in thin sections and areas with congested reinforcing, at the option of the CONTRACTOR and with written approval of the ENGINEER for the specific location.

### C.	Sitework Concrete:  Concrete to be used for curbs, gutters, catch basins, sidewalks, pavements, fence and guard post embedment, underground pipe encasement, underground duct bank encasement and all other concrete appurtenant to electrical facilities unless otherwise indicated.

### D.	Lean Concrete:  Concrete to be used for thrust blocks, pipe trench cut-off blocks and cradles that are indicated on the Drawings as unreinforced.  Lean concrete shall be used as protective cover for dowels intended for future connection.

## 1-04	SUBMITTALS:

### A.	Mix Designs:

#### 1.	Prior to beginning the WORK and within 14 days of the Notice to Proceed, the CONTRACTOR shall submit preliminary concrete mix designs which shall show the proportions and gradations of all materials proposed for each class and type of concrete.  Mix designs shall be checked by an independent testing laboratory acceptable to the ENGINEER.  All costs related to such checking shall be CONTRACTOR’S responsibility.

#### 2.	Test data relating to the cement, aggregate, and admixtures shall be less than six months old.  Furnish the submittals in accordance with ACI 301 for the following:

##### Mill tests for cement.

##### Admixture certification.  Chloride ion content shall be included.

##### Aggregate gradation test results and certification.

##### Delivery Tickets:

#### 3.	Where ready-mix concrete is used, the CONTRACTOR shall furnish delivery tickets at the time of delivery of each load of concrete.  Each ticket shall show the state certified equipment used for measuring and the total quantities, by weight, of cement, sand, each class of aggregate, admixtures, and the amounts of water in the aggregate added at the batching plant, and the amount allowed to be added at the Site for the specific design mix.  In addition, each ticket shall state the mix number, total yield in cubic yards, and the time of day, to the nearest minute, corresponding to the times when the batch was dispatched, when it left the plant, when it arrived at the Site, when unloading began, and when unloading was finished.

### B.	Other

#### 1.	The CONTRACTOR shall submit materials and methods for curing.

#### 2.	The CONTRACTOR shall submit product specifications, data, and installation instructions for all miscellaneous products called for in this specification.

## 1-05	QUALIFICATIONS:

A.	Truck mixers shall be equipped with electrically actuated counters by which the number of revolutions of the drum or blades may be readily verified.  The counter shall be of the resettable, recording type, and shall be mounted in the driver's cab.  The counters shall be actuated at the time of starting mixers at mixing speeds.

## 1-06	CERTIFICATIONS AND TESTING:

### A.	General

#### 1.	Concrete and other materials for testing shall be furnished by the CONTRACTOR, and the CONTRACTOR shall assist the ENGINEER in obtaining samples, and disposal and cleanup of excess material.

#### 2.	The testing laboratory will meet or exceed the requirements of ASTM C 1077.

#### 3.	The cost of trial batch, laboratory, and shrinkage tests on cement, aggregates, and concrete, will be the CONTRACTOR'S responsibility.

### B.	Trial Batch and Laboratory Tests

#### 1.	Tests for determining slump shall be in accordance with the requirements of ASTM C 143.

#### 2.	Testing for aggregate shall include sand equivalence, reactivity, organic impurities, abrasion resistance, and soundness, according to ASTM C 33.

#### 3.	A testing laboratory approved by the ENGINEER shall prepare a trial batch of each class of concrete, based on the preliminary concrete mixes submitted by the CONTRACTOR.  During the trial batch the aggregate proportions may be adjusted by the testing laboratory using the two coarse aggregate size ranges to obtain the required properties.  If one size range produces an acceptable mix, a second size range need not be used.  Such adjustments will be considered refinements to the mix design and will not be the basis for extra compensation to the CONTRACTOR.  Concrete shall conform to the requirements of this Section, whether the aggregate proportions are from the CONTRACTOR's preliminary mix design, or whether the proportions have been adjusted during the trial batch process.  The trial batch shall be prepared using the aggregates, cement, and admixture proposed for the project.  The trial batch materials shall be of a quantity such that the testing laboratory can obtain 3 drying shrinkage, and 6 compression test specimens from each batch.

#### 4.	The determination of compressive strength will be made by testing 6-inch diameter by 12-inch high cylinders; made, cured and tested in accordance with ASTM C 31 and ASTM C 39.  Three compression test cylinders will be tested at 7 days and 3 at 28 days.  The average compressive strength for the 3 cylinders tested at 28 days for any given trial batch shall not be less than 125 percent of the indicated compressive strength.

#### A sieve analysis of the combined aggregate for each trial batch shall be performed according to the requirements of ASTM C 136.  Values shall be given for percent passing each sieve.

### C.	Field Tests

#### 1.	The Contractor shall retain the services of an independent testing laboratory.

#### 2.	The CONTRACTOR shall pay the cost of any additional tests and investigation on WORK that does not meet the specifications.

#### 3.	Tests on pumped concrete shall be taken at the point of final placement.

#### 4.	Compressive Test:

##### Compressive test specimens shall be taken during construction from the first placement of each class of concrete placed each day and for each 150 cubic yards or fraction thereof each day.

##### Each set of test specimens shall consist of 5 cylinders.  Specimens shall be made in accordance with ASTM C 31.  Specimens shall be 6-inch diameter by 12-inch high cylinders.

##### Compression tests shall be performed in accordance with ASTM C 39.  Two (2) cylinders shall be broken at 7 days and two (2) at 28 days, and the remaining cylinder shall be held to verify test results, if needed.

##### The acceptance of the test results shall be the average of the strengths of the two specimens tested at 28 days as per ACI 318.  Evaluation and acceptance of the concrete shall be per ACI 318, Chapter 5.

#### 5.	Slump Tests:  One (1) slump test shall be taken per truckload in accordance with ASTM C 143.

#### Air Content:  Air content shall be determined for each compressive test taken in accordance with ASTM C 231 or by ASTM C 173.

#### Aggregate testing shall be made every 12 months during construction to insure continued compliance with these Specifications.

#### Concrete that fails to meet the ACI requirements and these Specifications is subject to removal and replacement.

## 1-07	WARRANTY:

### A.	Contractor shall supply 5-Year Maintenance Guarantee from the hardener manufacturer covering the hard, non-dusting qualities of the floor.

### B.	Contractor shall notify hardener manufacturer 3 working days prior to hardened concrete floor being placed.

### C.	Hardener manufacturer shall provide continuous supervision of concrete and hardener placements, supplying ENGINEER with a report of each day's placement.  Cost of supervision is to be borne by CONTRACTOR.

### D.	Contractor's final payment is not to be made until the guarantee is delivered to the Engineer.

# MATERIALS

## 2-01	GENERAL:

### All materials shall be classified as acceptable for potable water use according to NSF Standard 61.

### Cement for concrete that will contact potable water shall not be obtained from kilns that burn metal rich hazardous waste fuel.

### Materials shall be delivered, stored, and handled so as to prevent damage by water or breakage.  Cement reclaimed from cleaning bags or leaking containers shall not be used.  Cement shall be used in the sequence of receipt of shipments.

### Materials shall comply with the requirements of Sections 201, 203, and 204 of ACI 301, as applicable.

### Storage of materials shall conform to the requirements of Section 205 of ACI 301.

## 2-02	CEMENT:

### A.	Cement that provides sulfate resistance and that has a maximum water-cemtitious material ratio of 4 and minimum concrete strength (fe’).

### B.	Cement shall be standard Portland Cement Type II conforming to ASTM C 150.

### C.	A minimum of 85 percent of cement by weight shall pass a 325 screen.

### D.	A single brand of cement shall be used throughout the WORK, and prior to its use, the brand shall be accepted by the ENGINEER.

### E.	Sacked cement shall be stored in such a manner so as to permit access for inspection and sampling.  Certified mill test reports, including fineness, for each shipment of cement to be used shall be submitted to the ENGINEER, if requested, regarding compliance with these Specifications.

## 2-03	AGGREGATES:

### A.	Aggregates shall be obtained from pits acceptable to the ENGINEER, shall be non-reactive, and shall conform to the requirements of ASTM C 33.

### B.	When tested in accordance with ASTM C 33, the loss resulting after 5 cycles of the soundness test, shall not exceed 10 percent for fine aggregate and 12 percent for coarse aggregate, when using sodium sulfate.

### C.	When tested in accordance with ASTM C 33, the ratio of silica released to reduction in alkalinity shall not exceed 1.0.

### D.	Course Aggregates:

#### Coarse aggregates shall be crushed stone, gravel or other approved inert material having clean, hard, durable, uncoated particles conforming to ASTM C 33.

#### The coarse aggregates shall be prepared and handled in 2 or more size groups for combined aggregates with a maximum size greater than 3/4-inch.  When the aggregates are proportioned for each batch of concrete, the 2 size groups shall be combined.

#### When tested in accordance with ASTM C 33, the coarse aggregate shall show a loss not exceeding 42 percent after 500 revolutions, or 10.5 percent after 100 revolutions.

### E.	Fine Aggregates:

#### 1.	Fine aggregates shall be clean sand conforming to ASTM C 33.

#### 2.	When tested in accordance with ASTM D 2419, the sand equivalency shall not be less than 75 percent for an average of 3 samples, nor less than 70 percent for an individual test.  Gradation of fine aggregate shall conform to ASTM C 33 when tested in accordance with ASTM C 136 for the fineness modulus of the sand used.  The fineness modulus of sand used shall not be over 3.1.

#### 3.	When tested in accordance with ASTM C 33, the fine aggregate shall produce a color in the supernatant liquid no darker than the reference standard color solution.

## 2-04	WATER:

### A.	The water used in the concrete mix and for curing shall be clean, potable, and in accordance with ACI 318.  Water shall be free from objectionable quantities of silty organic matter, alkali, salts, and other impurities.

### B.	The water shall be considered potable, for the purposes of this Section only, if it meets the requirements of the local governmental agencies.  Agricultural water with high total dissolved solids (over 1000 mg/l TDS) shall not be used.

## 2-05	ADMIXTURES:

### General: All admixtures shall be compatible and be furnished by a single manufacturer capable of providing qualified field service representation.  Admixtures shall be used in accordance with manufacturer's recommendations.  If the use of an admixture is producing an inferior end result, the CONTRACTOR shall discontinue use of the admixture.  Admixtures shall not contain thiocyanates no more than 0.05 percent chloride ion, and shall be non-toxic after 30 days.

### B.	Air Entraining Admixtures:

#### 1.	Air entraining admixture shall conform to ASTM C 260.  Air content shall be tested at the point of placement.

#### 2.	The air-entraining agent shall be added to the batch in a portion of the mixing water.  The solution shall be batched by means of a mechanical batcher capable of accurate measurement.

### C.	Set Controlling and Water Reducing Admixtures:

#### 1.	Admixtures may be added at the CONTRACTOR's option, subject to the ENGINEER's approval, to control the set, effect water reduction, and increase workability.  The cost of adding an admixture shall be the CONTRACTOR's responsibility.  Concrete containing an admixture shall be first placed at a location determined by the ENGINEER.  Admixtures shall conform to the requirements of ASTM C 494.  The required quantity of cement shall be used in the mix regardless of whether or not an admixture is used.

#### 2.	Concrete shall not contain more than one water-reducing admixture.

#### 3.	Set retarding admixture may be either with or without water-reducing properties.  Where the air temperature at the time of placement is expected to be consistently greater than 80 degrees F, a set retarding admixture shall be used.  Set retarding admixture shall conform to ASTM C 494 Type B or D.

#### Set accelerating admixture may be either with or without water-reducing properties.  Where the air temperature at the time of placement is expected to be consistently less than 40 degrees F, a non-corrosive set accelerating admixture shall be used.  Set accelerating admixture shall conform to ASTM C 494 Type C or E.

#### Normal range water reducer shall conform to ASTM C 494, Type A.  The quantity of admixture used and the method of mixing shall be in accordance with the manufacturer's instructions and recommendations.

#### High range water reducer shall conform to ASTM C 494, Type F or G. High range water reducer shall be added to the concrete after all other ingredients have been mixed and initial slump has been verified.  No more than 14 ounces of water reducer per sack of cement shall be used.  Water reducer shall be considered as part of the mixing water when calculating the water/cement ratio.

##### If the high range water reducer is added to the concrete at the Site, it may be used in conjunction with the same water reducer added at the batch plant.  Concrete shall have a slump of 3 inches plus or minus 1/2-inch prior to adding the high range water reducing admixture at the Site.  The high range water reducing admixture shall be accurately measured and pressure injected into the mixer as a single dose by an experienced technician.  A standby system shall be provided and tested prior to each day's operation of the primary system.

##### Concrete shall be mixed at mixing speed for a minimum of 70 mixer revolutions or 5 minutes after the addition of the high range water reducer, unless recommended otherwise by the manufacturer.

## 2-06	CURING MATERIALS:

### A.	Curing compound shall conform to ASTM C 309, Type I.  Curing compound shall be white pigmented, resin based and compliant with local VOC requirements.  When curing compound must be removed for finishes or grouting, it shall be of a dissipating type.  Sodium silicate compounds shall not be allowed.

### B.	Polyethylene sheet for use as concrete curing blanket shall be white and shall have a nominal thickness of 6 mils.  The loss of moisture when determined in accordance with the requirements of ASTM C 156, shall not exceed 0.055 grams per square centimeter of surface.

### C.	Polyethylene-coated waterproof paper sheeting for use as concrete curing blanket shall consist of white polyethylene sheeting free of visible defects, uniform in appearance, have a minimum thickness of 2 mils, and be permanently bonded to waterproof paper conforming to the requirements of Federal Specification UU-B-790A.  The loss of moisture, when determined in accordance with the requirements of ASTM C 156, shall not exceed 0.055 gram per square centimeter of surface.

### D.	Polyethylene-coated burlap for use as concrete curing blanket shall be minimum 4-mil thick, white opaque polyethylene film impregnated or extruded into one side of the burlap.  Burlap shall weigh not less than 9 ounces per square yard.  The loss of moisture, when determined in accordance with the requirements of ASTM C 156, shall not exceed 0.055 grams per square centimeter of surface.

### E.	Curing mats for use in Curing Method 6 below, shall be heavy shag rugs or carpets or cotton mats quilted at 4-inches on center.  Curing mats shall weigh a minimum of 12 ounces per square yard when dry.

## 2-07	MISCELLANEOUS MATERIALS:

### A.	Damp proofing agent shall be an asphalt emulsion conforming to ASTM D1227, Type III, Class 1.

### B.	Evaporation retardant shall create a monomolecular film on the concrete.  The retardant shall have no effect on cement hydration and shall meet local VOC requirements.  Evaporation retardant shall not effect adhesion of curing compounds or other treatments and shall not effect the color of the concrete.

### C.	Floor hardener shall be provided at the following locations:

#### 1.	The operating deck only.

#### 2.	Surface hardener shall be system of integral hardening admixture, shake-on hardener, and surface applied liquid hardener acceptable for both indoor and outdoor applications.

#### 3.	Integral hardening admixture shall be a non-chloride admixture that will produce high strength, durable, hard, non-dusting abrasive resistant concrete.  Admixture shall react with the portland cement to produce more complete hydration.

#### 4.	Shake-on hardener shall be a ready to use, heavy-duty emery shake-on floor hardener.  Aggregate shall consist of no less than 35% aluminum oxide.  Hardener shall produce a hard, non-slip floor that will not rust.

#### 5.	Surface applied liquid hardener shall be a water borne chemical hardener and dustproofer.  Hardener shall meet local VOC requirements and shall be a solution of silicates and fluosilicates.

### D.	Reinforcement shall be per SECTION 03200 Concrete Reinforcement.

### E.	Water Stops shall be per SECTION 03100 Concrete Formwork.

### Damp proofing agent shall be a waterborne emulsified-asphalt.  Damp proofing shall be suitable for "green" or slightly damp surfaces and shall withstand normal expansion and contraction of the concrete.  Damp proofing agent shall breath to allow vapors to escape.  Damp proofing agent shall meet local VOC requirements.

### Bonding agents shall be 100% solids, epoxy adhesives conforming to the following:

#### For bonding freshly-mixed, plastic concrete to hardened concrete, bonding agent shall be a medium viscosity adhesive conforming to ASTM C 881 Type II, Grade 2, Class C,

#### For bonding hardened concrete or masonry to steel, bonding agent shall be a non-sagging gel adhesive conforming to ASTM C 881 Type I or IV, Grade 3, Class C.

### Vapor Barrier:

#### Vapor Barrier shall consist of a composite of heavy kraft paper, asphalt, fiberglass reinforcement, and polyethylene film.  The composite shall be laminated under heat and pressure.

#### Vapor Barrier shall comply with federal specification UU-B-790A, Type I, Grade A, Style 4.  Vapor Barrier shall have a water vapor permeance of less than 0.30 perms when tested per ASTM E 96.

#### Vapor Barrier shall be installed under concrete slabs of all habitable spaces.  Barrier shall be installed per the manufacturer recommendations and per ASTM E 1643.

### Non-Waterstop Joint Material:

#### Preformed Joint Material:  Preformed asphalt-impregnated fiber conforming to ASTM D 1751.

#### Bond Breaker:  All bond breakers shall be roofing felt or 15 mils minimum dry film thickness of bituminous paint as indicated.

## 2-08	CONCRETE DESIGN REQUIREMENTS:

### A.	General:  Concrete shall be composed of cement, admixtures, aggregates, and water of the qualities indicated.  The exact proportions in which these materials are to be used for different parts of the WORK will be determined during the trial batch process.  In general, the mix shall be designed to produce a concrete capable of being deposited so as to obtain maximum density and minimum shrinkage, and, where deposited in forms, to have good consolidation properties and maximum smoothness of surface.  The aggregate gradations shall be formulated to provide fresh concrete that will not promote rock pockets around reinforcing steel or embedded items.  The proportions shall be changed whenever necessary or desirable to meet the required results.  All changes shall be subject to review by the ENGINEER.

### B.	Fine Aggregate Composition:

#### 1.	In mix designs for structural concrete, the percentage of fine aggregate in total aggregate by weight, shall be as indicated in the following table.

| Fine Aggregate |  |
| --- | --- |
| Fineness Modulus | Maximum Percent |
| 2.7 or less | 41 |
| 2.7 to 2.8 | 42 |
| 2.8 to 2.9 | 43 |
| 2.9 to 3.1 | 44 |

#### 2.	For other concrete, the maximum percentage of fine aggregate of total aggregate, by weight, shall not exceed 50.

### C.	Duct bank concrete shall contain an integral red-oxide coloring pigment.  Concrete shall be dyed red throughout. Surface treatment to color duct banks will not be acceptable.

### D.	Water/Cement Ratio and Compressive Strength:

#### 1.	Water/cement ratio is given for aggregates in saturated-surface dry condition and total moisture of all aggregates, calculated by ASTM C 566, less the absorption of the aggregate as calculated by ASTM C 127 and C 128 shall represent total free moisture in the aggregate determine the water/cement ratio, total free moisture of aggregates shall be added to batch water to estimate water content of concrete.  Concrete shall have the following minimum properties:

### E.	Concrete Proportions:

1.	The proportions of concrete materials shall follow the Table as shown or use ACI 211

| Type of Work | Min 28-Day Compressive Strength (psi) | Maximum Size Aggregate (in) | Cement Content per cubic yd (lbs) | Maximum W/C Ratio (by weight) |
| --- | --- | --- | --- | --- |
| Structural Concrete |  |  |  |  |
| Roof, floor slabs, columns, walls, and all other concrete items not indicated elsewhere. | 4,000 | 1 | 674 to 710 | 0.4 |
| Pea Gravel Concrete |  |  |  |  |
| Thin sections and areas with congested reinforcing, at the CONTRACTOR'S option and with the written approval of the ENGINEER for The specific location. | 4,000 | 3/8 | 752 to 788 | 0.40 |
| Sitework concrete | 3,500 | 1 | 470 (min) | 0.50 |
| Lean concrete | 2,000 | 1 | 376 (min) | 0.60 |

NOTE:  The CONTRACTOR is cautioned that the limiting parameters above are not a mix design. Admixtures may be required to achieve workability required by the CONTRACTOR'S construction methods and aggregates.  The CONTRACTOR is responsible for providing concrete with the required workability and strength.

### F.	Adjustments to Mix Design:  The CONTRACTOR may elect to decrease the water/cement ratio to achieve the strength and shrinkage requirements and/or add water reducers, as required to achieve workability.  The mixes shall be changed whenever such change is necessary or desirable to secure the required strength, density, workability, and surface finish, and the CONTRACTOR shall be entitled to no additional compensation because of such changes.  Any changes to the accepted concrete mix design shall be submitted to the ENGINEER for review and shall be tested again in accordance with these Specifications.

### G.	When using a floor hardener the water/cement ratio shall not be greater than specified by the hardener manufacturer.

## 2-09	CONSISTENCY:

### A.	The quantity of water in a batch of concrete shall be just sufficient, with a normal mixing period, to produce a concrete which can be worked properly into place without segregation and which can be compacted by vibratory methods to give the desired density, impermeability, and smoothness of surface.  The quantity of water shall be changed as necessary, with variations in the nature or moisture content of the aggregates, to maintain uniform production of a desired consistency.  The consistency of the concrete in successive batches shall be determined by slump tests in accordance with ASTM C 143  The slumps shall be as follows:

| Part of Work | Slump (in) |
| --- | --- |
| All concrete, unless indicated otherwise | 3-inches plus or minus 1-inch |
| With high range water reducer added | 7-inches plus or minus 2-inches |
| Pea gravel mix | 7-inches plus or minus 2-inches |
| Ductbank and pipe encasement | 5-inches plus or minus 1-inch |
| Concrete with hardener | Per Hardener Manufacturer |

## 2-10	MEASUREMENT:

### A.	The amount of cement and of each separate size of aggregate entering into each batch of concrete shall be determined by direct weighing equipment furnished by the CONTRACTOR and acceptable to the ENGINEER.

### B.	Weighing tolerances

| Material | Percent of Total Weight |
| --- | --- |
| Cement | 1 |
| Aggregates | 3 |
| Admixtures | 3 |

### C.	The quantity of water entering the mixer shall be measured by a suitable water meter or other measuring device of a type acceptable to the ENGINEER and capable of measuring the water in variable amounts within a tolerance of one percent.  The water feed control mechanism shall be capable of being locked in position so as to deliver constantly any required amount of water to each batch of concrete.  A positive quick-acting valve shall be used for a cut-off in the water line to the mixer.  The operating mechanism shall prevent leakage when the valves are closed.

# EXECUTION

## 3-01	PROPORTIONING AND MIXING:

### Proportioning of the mix shall conform to the requirements of Chapter 3 "Proportioning" of ACI 301.

### Mixing shall conform to the requirements of Chapter 7 of ACI 301.

### Slumps shall be as indicated herein.

### Retempering of concrete or mortar that has partially hardened shall not be permitted.

## 3-02	PREPARATION OF SURFACES FOR CONCRETING:

### A.	General:  Earth surfaces shall be thoroughly wetted by sprinkling prior to the placing of any concrete, and these surfaces shall be kept moist by frequent sprinkling up to the time of placing concrete thereon.  The surface shall be free from standing water, mud, and debris at the time of placing concrete.

### B.	Vapor Barrier

#### Vapor Barrier shall be installed under on-grade building floor slabs of occupiable (non-hydraulic) structures and at other locations indicated.

#### Base shall be leveled, compacted, and tamped per SECTION 02200 Earthwork.  Remove sharp edges, projection materials and roughness that might penetrate vapor barrier.  Install barrier with width parallel with the direction of the pour of the concrete.

#### Place, protect, and repair defects in sheet according to ASTM E 1643, and the manufacturer's written instructions.  Seams shall be lapped and sealed in accordance with ASTM E 1643.

#### The CONTRACTOR shall exercise care to avoid puncturing or tearing the vapor barrier during installation.  Patch punctures and tears as they occur.

### C.	Joints in Concrete:

#### 1.	All joints shall be installed where indicated on the Drawings or where otherwise approved by the Engineer.  The surface of the construction joint shall be rough and prior to placement shall be cleaned and moistened with water.

#### 2.	Concrete surfaces upon or against which concrete is to be placed, where the placement of the concrete has been stopped or interrupted so that, as determined by the ENGINEER, the new concrete cannot be incorporated integrally with that previously placed, are defined as construction joints.  The surfaces of horizontal joints shall be given a compacted, roughened surface for good bonding.  Except where the Drawings call for joint surfaces to be coated, the joint surfaces shall be cleaned of all laitance, loose or defective concrete, foreign material, and be roughened to a minimum 1/4-inch amplitude.  Such cleaning and roughening shall be accomplished by hydroblasting or sandblasting (exposing aggregate) followed by thorough washing.  Pools of water shall be removed from the surface of construction joints before the new concrete is placed.

#### 3.	After the surfaces have been prepared, all approximately horizontal construction joints shall be covered with a 6-inch lift of a pea gravel mix.  The mix shall be placed and spread uniformly.  Wall concrete shall follow immediately and shall be placed upon the fresh pea gravel mix.  If high range water reducer is used in the wall concrete, then the pea gravel joint topping does not need to be used.

### D.	Placing Interruptions:  When placing of concrete is to be interrupted long enough for the concrete to take a set, the working face shall be given a shape by the use of forms or other means that will secure proper union with subsequent work; provided that construction joints shall be made only where acceptable to the ENGINEER.

### Embedded Items:

#### No concrete shall be placed until all formwork, installation of parts to be embedded, reinforcement steel, and preparation of surfaces involved in the placing have been completed and accepted by the ENGINEER at least 4 hours before placement of concrete.  Surfaces of forms and embedded items that have become encrusted with dried grout from previous usage shall be cleaned before the surrounding or adjacent concrete is placed.

#### Inserts or other embedded items shall conform to the requirements herein.

#### Reinforcement, anchor bolts, sleeves, inserts, and similar items shall be set and secured in the forms at locations indicated on the Drawings or shown by Shop Drawings and shall be acceptable to the ENGINEER before any concrete is placed.  Accuracy of placement is the responsibility of the CONTRACTOR.

### Casting New Concrete Against Old:  Where concrete is to be cast against old concrete (any concrete which is greater than 60 days of age), the surface of the old concrete shall be thoroughly cleaned and roughened by hydroblasting or sandblasting to expose aggregate.  The joint surface shall be coated with an epoxy bonding agent unless indicated otherwise by the ENGINEER.

### No concrete shall be placed in any structure until all water entering the space to be filled with concrete has been properly cut off or has been diverted by pipes, or other means, and carried out of the forms, clear of the WORK.  No concrete shall be deposited underwater nor shall the CONTRACTOR allow still water to rise on any concrete until the concrete has attained its initial set.  Water shall not be permitted to flow over the surface of any concrete in such manner and at such velocity as will injure the surface finish of the concrete.  Pumping or other necessary dewatering operations for removing ground water, if required, shall be subject to the review of the ENGINEER.

### Corrosion Protection:  Pipe, conduit, dowels, and other ferrous items required to be embedded in concrete construction shall be so positioned and supported prior to placement of concrete that there will be a minimum of 2-inches clearance between said items and any part of the concrete reinforcement.  Securing such items in position by wiring or welding them to the reinforcement will not be permitted.

### Openings for pipes, inserts for pipe hangers and brackets, and anchors shall, where practicable, be provided during the placing of concrete.

### Anchor bolts shall be accurately set and shall be maintained in position by templates while being embedded in concrete.

### Cleaning:  The surfaces of metalwork to be in contact with concrete shall be thoroughly cleaned of all dirt, grease, loose scale and rust, grout, mortar, and other foreign substances immediately before the concrete is placed.

## 3-03	CONVEYING:

### A.	Concrete shall be conveyed from the mixer to the place of final deposit by methods that will prevent separation or loss of material.

### B.	No aluminum materials shall be used in conveying any concrete.

### C.	Ends of chutes, hopper gates, and all other points of concrete discharge throughout the CONTRACTOR'S conveying, hoisting, and placing system shall be designed and arranged so that concrete passing from them will not fall separated into whatever receptacle immediately receives it.  Conveyor belts, if used, shall be of a type acceptable to the ENGINEER.  Chutes longer than 50 feet will not be permitted.  Minimum slopes of chutes shall be such that concrete of the indicated consistency will readily flow in them.  If a conveyor belt is used, it shall be wiped clean by a device operated in such a manner that none of the mortar adhering to the belt will be wasted.  All conveyor belts and chutes shall be covered.

### D.	Pumping:

#### If the pumped concrete does not produce satisfactory end results, the CONTRACTOR shall discontinue the pumping operation and proceed with the placing of concrete using conventional methods.

#### The pumping equipment shall have 2 cylinders and be designed to operate with one cylinder in case the other one is not functioning.  In lieu of this requirement, the CONTRACTOR may have a standby pump on the Site during pumping.

#### The minimum diameter of the hose conduits shall be in accordance with ACI 304.

#### Pumping equipment and hose conduits that are not functioning properly shall be replaced.

#### Aluminum conduits for conveying the concrete shall not be permitted.

#### Concrete samples for slump, air content, and test cylinders will be taken at the placement end of the hose.

## 3-04	DELIVERY:

### A.	Ready-mixed concrete shall be batched, mixed, transported and delivered in accordance with these specifications and ASTM C 94 including the following supplementary requirements.

B.	Ready mixed concrete for the flood wall shall be as follow:

#### 1.	Concrete shall be discharged within 1-1/2 hours from the time concrete was mixed, if centrally mixed, or from the time the original water was added, if transit-mixed or before the drum has been revolved 300 revolutions, whichever is first.

#### 2.	Truck mixers and their operation shall be such that the concrete throughout the mixed batch as discharged is within acceptable limits of uniformity with respect to consistency, mix, and grading.  If slump tests taken at approximately the 1/4 and 3/4 points of the load during discharge give slumps differing by more than one-inch when the required slump is 3-inches or less, or if they differ by more than 2-inches when the required slump is more than 3-inches, the mixer shall not be used on the WORK unless the causative condition is corrected and satisfactory performance is verified by additional slump tests.  Mechanical details of the mixer, such as water measuring and discharge apparatus, condition of the blades, speed of rotation, general mechanical condition of the unit, and clearance of the drum, shall be checked before a further attempt to use the unit will be permitted.

#### 3.	The use of non-agitating equipment for transporting ready-mixed concrete will not be permitted.  The quality and quantity of materials used in ready-mixed concrete and in batch aggregates shall be subject to continuous inspection at the batching plant by the ENGINEER.

#### 4.	Each batch of concrete shall be mixed in a truck mixer for not less than 70 revolutions of the drum or blades at the rate of rotation designated by the manufacturer of equipment.  Additional mixing, if any, shall be at the speed designated by the manufacturer of the equipment as agitating speed.  All materials including mixing water shall be in the mixer drum before actuating the revolution counter for determining the number of revolution of mixing.

## 3-05	PLACING:

### A.	Non-Conforming Work or Materials:  Concrete which during or before placing is found not to conform to the requirements indicated herein shall be rejected and immediately removed from the WORK.  Concrete which is not placed in accordance with these Specifications, or which is of inferior quality, shall be removed and replaced.

### B.	Unauthorized Placement:  No concrete shall be placed except in the presence of a duly authorized representative of the ENGINEER.  The CONTRACTOR shall notify the ENGINEER in writing at least 24 hours in advance of placement of any concrete.

### C.	Concrete shall not be dropped more than four feet (4’) without use of chutes or tremies.  Concreting shall be a continuous operation until placement of the section is complete.  All concrete shall be worked around reinforcement and embedded items.  If vibrators are used, care shall be taken not to segregate concrete.  Vibrators will not be allowed to move concrete within the form.  All forms and subgrade shall be dampened prior to placement and excess water removed.

### D.	Placing of concrete shall conform to the applicable requirements of Chapter 8 of ACI 301 and the requirements of this Section.

### E.	Placement in Slabs:  Concrete placed in sloping slabs shall proceed uniformly from the bottom of the slab to the top, for the full width of the placement.  As the work progresses, the concrete shall be vibrated and carefully worked around the slab reinforcement, and the surface of the slab shall be screeded in an up-slope direction.

### F.	Concrete shall not be dropped through reinforcement steel or into any deep form, nor shall concrete be placed in any form in such a manner as to leave accumulation of mortar on the form surfaces above the placed concrete.  In such cases, means such as hoppers and, if necessary, vertical ducts of canvas, rubber, or metal shall be used for placing concrete in the forms in a manner that it may reach the place of final deposit without separation.  Concrete shall be uniformly distributed during the process of depositing and in no case after depositing shall any portion be displaced in the forms more than 6-feet in horizontal direction.  Concrete in wall forms shall be deposited in uniform horizontal layers not deeper than 2-feet; and care shall be taken to avoid inclined layers or inclined construction joints except where such are required for sloping members.  Each layer shall be placed while the previous layer is still soft.  The rate of placing concrete in wall forms shall not exceed 5-feet of vertical rise per hour.  Sufficient illumination shall be provided in the interior of all forms so that the concrete at the places of deposit is visible from the deck or runway.

### G.	Concrete with hardener shall be placed per the hardener manufacturers written recommendations.

### H.	Casting New Concrete Against Old:  Epoxy adhesive bonding agent shall be applied to the old surfaces according to the manufacturer's written recommendations.  This provision shall not apply to joints where waterstop is provided, see SECTION 03100 Concrete Formwork.

### I.	Temperature of Concrete: The temperature of concrete when it is being placed shall be not more than 90 degrees F nor less than 55 degrees F for sections less than 12-inches thick nor less than 50 degrees for all other sections.  The CONTRACTOR shall be entitled to no additional compensation on account of the temperature requirements.

### J.	Hot Weather Placement

#### 1.	Placement of concrete in hot weather shall conform to ACI 305 and the following:

#### 2.	When the temperature of the concrete is 85 degrees F or above, the time between the introduction of the cement to the aggregates and discharge shall not exceed [60] minutes.

#### 3.	If concrete is placed when the weather is such that the temperature of the concrete would exceed 90 degrees F, the CONTRACTOR shall employ effective means, such as pre-cooling of aggregates and mixing water using ice or placing at night, as necessary to maintain the temperature of the concrete below 90 degrees F as it is placed.

### K.	Cold Weather Placement

#### 1.	Placement of concrete in cold weather shall conform to ACI 306.1, and the following:

#### 2.	Concrete ingredients shall not be heated to a temperature higher than that necessary to keep the temperature of the mixed concrete, as placed, from falling below the minimum temperature.

#### 3.	Remove all snow, ice, and frost from the surfaces, including reinforcement, against which concrete is to be placed.  Before beginning concrete placement, thaw the subgrade to a minimum depth of 6-inches.  Reinforcement and embedded items shall be warmed to above 32 degrees F prior to concrete placement.

#### 4.	Maintain the concrete temperature above 50 degrees F for at least 72 hours after placement.

### Order of Placing Concrete

#### The order of placing concrete in all parts of the WORK shall be acceptable to the ENGINEER.  In order to minimize the effects of shrinkage, the concrete shall be placed in units as bounded by construction joints at the indicated locations.  The placing of units shall be done by placing alternate units in a manner such that each unit placed shall have cured at least 5 days for hydraulic structures and 2 days for all other structures before the contiguous unit or units are placed, except that the corner sections of vertical walls shall not be placed until the 2 adjacent wall panels have cured at least 10 days for hydraulic structures and 4 days for all other structures.

#### The surface of the concrete shall be level whenever a run of concrete is stopped.  To insure a level, straight joint on the exposed surface of walls, a wood strip at least 3/4-inch thick shall be tacked to the forms on these surfaces.  The concrete shall be carried about 1/2-inch above the underside of the strip.  About one hour after the concrete is placed, the strip shall be removed and any irregularities in the edge formed by the strip shall be leveled with a trowel and laitance shall be removed.

3-06	TAMPING AND VIBRATING:

### A.	As concrete is placed in the forms or in excavations, it shall be thoroughly settled and compacted throughout the entire depth of the layer which is being consolidated, into a dense, homogeneous mass, filling all corners and angles, thoroughly embedding the reinforcement, eliminating rock pockets, and bringing only a slight excess of water to the exposed surface of concrete.  Vibrators shall be Group 3 per ACI 309, high speed power vibrators (8000 to 12,000 rpm) of an immersion type in sufficient number and with at least one standby unit as required.  Group 2 vibrators may be used only at specific locations when accepted by the ENGINEER.

### B.	Care shall be used in placing concrete around waterstops.  The concrete shall be carefully worked by rodding and vibrating to make sure that all air and rock pockets have been eliminated.  Where flat-strip type waterstops are placed horizontally, the concrete shall be worked under the waterstops by hand, making sure that air and rock pockets have been eliminated.  Concrete surrounding the waterstops shall be given additional vibration over and above that used for adjacent concrete placement to assure complete embedment of the waterstops in the concrete.

### C.	Concrete in walls shall be internally vibrated and at the same time rammed, stirred, or worked with suitable appliances, tamping bars, shovels, or forked tools until it completely fills the forms or excavations and closes snugly against all surfaces.  Subsequent layers of concrete shall not be placed until the layers previously placed have been worked thoroughly.  Vibrators shall be provided in sufficient numbers, with standby units as required, to accomplish the required results within 15 minutes after concrete of the prescribed consistency is placed in the forms.  The vibrating head shall not contact the surfaces of the forms.  Care shall be taken not to vibrate concrete excessively or to work it in any manner that causes segregation of its constituents.

## 3-07	CURING AND DAMPPROOFING:

## A.	Concrete shall be cured for a minimum of five (5) days after placement in accordance with the methods indicated below for the different parts of the WORK.

| Surface to be Cured or Dampproofed | Method |
| --- | --- |
| Unstripped forms | 1 |
| Wall sections with forms removed | 6 |
| Construction joints between footings and walls, and between floor slab and columns | 2 |
| Encasement and ductbank concrete and thrust blocks | 3 |
| All concrete surfaces not specifically indicated in this Paragraph | 4 |
| Floor slabs on grade in hydraulic structures | 5 |
| Slabs not on grade | 6 |
| Concrete with hardener | Per manufacture's written recommendations |

### 1.	Method 1:  Wooden forms shall be wetted immediately after concrete has been placed and shall be kept wet with water until removal.  If steel forms are used the exposed concrete surfaces shall be kept continuously wet until the forms are removed.  If forms are removed within 7 days of placing the concrete, curing shall be continued in accordance with Method 6 below.

### 2.	Method 2:  The surface shall be covered with burlap mats which shall be kept wet with water for the duration of the curing period, until the concrete in the walls has been placed.  No curing compound shall be applied to surfaces cured under Method 2.

### 3.	Method 3:  The surface shall be covered with moist earth not less than 4 hours nor more than 24 hours after the concrete is placed.  Earthwork operations that may damage the concrete shall not begin until at least 7 days after placement of concrete.

### 4.	Method 4:  The surface shall be sprayed with a liquid curing compound.

#### a.	It shall be applied in accordance with the manufacturer's printed instructions at a maximum coverage rate of 200 square feet per gallon and in such a manner as to cover the surface with a uniform film that will seal thoroughly.

#### b.	Where the curing compound method is used, care shall be exercised to avoid damage to the seal during the 7-day curing period.  If the seal is damaged or broken before the expiration of the curing period, the break shall be repaired immediately by the application of additional curing compound over the damaged portion.

#### c.	Wherever curing compound has been applied by mistake to surfaces against which concrete subsequently is to be placed and to which it is to adhere, compound shall be entirely removed by wet sandblasting just prior to the placing of new concrete.

#### d.	Curing compound shall be applied as soon as the concrete has hardened enough to prevent marring on unformed surfaces and within 2 hours after removal of forms.  Repairs to formed surfaces shall be made within the 2 hour period; provided, however, that any such repairs which cannot be made within the said 2 hour period shall be delayed until after the curing compound has been applied.  When repairs are to be made to an area on which curing compound has been applied, the area involved shall first be wet-sandblasted to remove the curing compound.

#### e.	At locations where concrete is placed adjacent to a panel which has been coated with curing compound, the panel shall have curing compound reapplied to an area within 6-feet of the joint and to any other location where the curing membrane has been disturbed.

#### f.	Prior to final acceptance of the WORK, all visible traces of curing compound shall be removed from all surfaces in such a manner that does not damage the surface finish.

### 5.	Method 5:

#### a.	Until the concrete surface is covered with curing compound, the entire surface shall be kept damp by applying water using nozzles that atomize the flow so that the surface is not marred or washed.  The concrete shall be given a coat of curing compound in accordance with Method 4 above.  Not less than one hour nor more than 4 hours after the curing compound has been applied, the surface shall be wetted with water delivered through a fog nozzle, and concrete-curing blankets shall be placed on the slabs.  The curing blankets shall be polyethylene sheet, polyethylene-coated waterproof paper sheeting, or polyethylene-coated burlap.  The blankets shall be laid with the edges butted together and with the joints between strips sealed with 2-inch wide strips of sealing tape or with edges lapped not less than 3-inches and fastened together with a waterproof cement to form a continuous watertight joint.

#### b.	The curing blankets shall be left in place during the 7-day curing period and shall not be removed until after concrete for adjacent work has been placed.  If the curing blankets become torn or otherwise ineffective, the CONTRACTOR shall replace damaged sections.  During the first 3 days of the curing period, no traffic of any nature and no depositing, temporary or otherwise, of any materials shall be permitted on the curing blankets.  During the remainder of the curing period, foot traffic and temporary depositing of materials that impose light pressure will be permitted only on top of plywood sheets 5/8-inch minimum thickness, laid over the curing blanket.  The CONTRACTOR shall add water under the curing blanket as often as necessary to maintain damp concrete surfaces at all times.

### 6.	Method 6:  This method applies to both walls and slabs.

#### a.	The concrete shall be kept continuously wet by the application of water for a minimum period of at least 7 consecutive days beginning immediately after the concrete has reached final set or forms have been removed.

#### b.	Until the concrete surface is covered with the curing medium, the entire surface shall be kept damp by applying water using nozzles that atomize the flow so that the surface is not marred or washed.

#### c.	Heavy curing mats shall be used as a curing medium to retain the moisture during the curing period.  The curing medium shall be weighted or otherwise held substantially in contact with the concrete surface to prevent being dislodged by wind or any other causes.  Edges shall be continuously held in place.

#### d.	The curing blankets and concrete shall be kept continuously wet by the use of sprinklers or other means both during and after normal working hours.

#### e.	Immediately after the application of water has terminated at the end of the curing period, the curing medium shall be removed, the entire concrete surface shall be wetted, and curing compound shall be immediately applied to the entire surface in accordance with Method 4 above.

#### f.	The CONTRACTOR shall dispose of excess water from the curing operation to avoid damage to the WORK.

### 7.	Damp proofing

#### a.	The exterior surfaces of backfilled dry well walls and buried roof slabs shall be damp proofed as follows.

#### b.	Immediately after completion of curing the surface shall be sprayed with a damp proofing agent consisting of an asphalt emulsion.  Application shall be in 2 coats.  The first coat shall be diluted to one-half strength by the addition of water and shall be sprayed on so as to provide a maximum coverage rate of 100 square feet per gallon of dilute solution.  The second coat shall consist of an application of the undiluted material, and shall be sprayed on so as to provide a maximum coverage rate of 100 square feet per gallon.  Damp proofing material shall be as indicated above.

#### c.	As soon as the material has taken an initial set, the entire area thus coated shall be coated with whitewash.  Any formula for mixing the whitewash may be used if it produces a uniformly coated white surface and remains until placing of the backfill.  If the whitewash fails to remain on the surface until the backfill is placed, the CONTRACTOR shall apply additional whitewash.

## 3-08	CONCRETE FINISHES:

### A.	General:  Surfaces shall be free from fins, bulges, ridges, offsets, honeycombing, or roughness of any kind, and shall present a finished, smooth, continuous hard surface.  Allowable deviations from plumb or level and from the alignment, profiles, and dimensions indicated are defined as tolerances and are indicated herein.  These tolerances are to be distinguished from irregularities in finish as described herein.  Aluminum finishing tools shall not be used.

### B.	Formed Surfaces:

#### 1.	No treatment is required after form removal except for curing, repair of defective concrete, and treatment of surface defects.  Where architectural finish is required, it shall be as indicated.

### C.	Unformed Surfaces:

#### 1.	After proper and adequate vibration and tamping, unformed top surfaces of slabs, floors, walls, and curbs shall be brought to a uniform surface with suitable tools.  Immediately after the concrete has been screeded, it shall be treated with a liquid evaporation retardant.  The retardant shall be used again after each work operation as necessary to prevent drying shrinkage cracks.  The classes of finish for unformed concrete surfaces are designated and defined as follows:

| Unformed Surface Finish Schedule |  |
| --- | --- |
| Area | Finish |
| Grade slabs and foundations to be covered with concrete or fill material | U1 |
| Floors to be covered with grouted tile or topping grout | U2 |
| Water bearing slabs with slopes 10 percent and less | U3 |
| Water bearing slabs with slopes greater than 10 percent | U4 |
| Slabs not water bearing | U4 |
| Slabs to be covered with built-up roofing | U2 |
| Interior slabs and floors to receive architectural finish | U3 |
| Top surface of walls subject to foot traffic | U4 |
| Top surface of walls not subject to foot traffic | U3 |
| Floors to receive surface hardener | U5 |

#### 2.	Finish U1 - Sufficient leveling and screeding to produce an even, uniform surface with surface irregularities not to exceed 3/8-inch.  No further special finish is required.

#### 3.	Finish U2 - (Float Finish)

##### Compact, accurately screed and float to a true uniform surface.

##### Surfaces shall be floated with wood or metal floats or a finishing machine using float blades.  Excessive floating of surfaces while the concrete is plastic and dusting of dry cement and sand on the concrete surface to absorb excess moisture will not be permitted.

##### Floating shall be the minimum necessary to produce a surface that is free from screed marks and is uniform in texture.  Test surface with straightedge and eliminate high and low spots of more than 1/8 inch in 10 feet.  Surface irregularities shall not exceed 1/4-inch.

##### Joints and edges shall be tooled where indicated or as determined by the ENGINEER.

#### 4.	Finish U3 - (Hand-Troweled Finish)

##### a.	Finish surface as in Finish U2 - Float Finish and after the surface has hardened sufficiently to prevent excess of fine material from being drawn to the surface, trowel with steel trowel to obtain a smooth dense finish after concrete has hardened to ring under the trowel.

##### b.	The finish shall be produce a smooth dense uniform surface free of all irregularities, blemishes, ripples, and trowel marks.

#### 5.	Finish U4 - (Nonskid Finish)

##### a.	Trowel the Finish U3 - Hand-trowelled Finish surface to remove local depressions or high points.  In addition, the surface shall be given a light broom finish with brooming perpendicular to drainage unless otherwise indicated.

##### b.	The resulting surface shall be rough enough to provide a nonskid finish.

#### 6.	Finish U5 - (Surface hardener)

##### a.	Immediately after screeding, shake on hardener shall be applied per the manufacturer's written recommendations.

##### b.	Surface shall receive a minimum of two coats of a liquid hardener per the manufacturer's written recommendations.

## 3-09	PROTECTION:

### A.	The CONTRACTOR shall protect concrete against injury until final acceptance.

### B.	Fresh concrete shall be protected from damage due to rain, hail, sleet, or snow.  The CONTRACTOR shall provide such protection while the concrete is still plastic and whenever precipitation is imminent or occurring.

## 3-10	DEFECTIVE SURFACE TREATMENTS:

### A.	Patching Concrete:

#### 1.	Patch all tie holes, honeycombs or other defects with a Portland Cement and sand grout.

#### 2.	Defective surfaces to be repaired shall be cut back from trueline a minimum depth of 1/2-inch over the entire area.  Feathered edges will not be permitted.  Where chipping or cutting tools are not required in order to deepen the area properly, the surface shall be prepared for bonding by the removal of all laitance or soft material, plus not less than 1/32-inch depth of the surface film from all hard portions by means of an efficient sandblast.

#### 3.	After cutting and sandblasting, the surface shall be wetted sufficiently in advance of shooting with shotcrete or with cement mortar so that while the repair material is being applied, the surfaces underneath will remain moist but not so wet as to overcome the suction upon which a good bond depends.

#### 4.	Holes left by tie-rod cones shall be reamed with suitable toothed reamers so as to leave the surfaces of the holes clean and rough.  Holes then shall be repaired in an approved manner with dry-packed cement grout.  Holes left by form-tying devices having a rectangular cross-section and other imperfections having a depth greater than their least surface dimension shall not be reamed but shall be repaired in an approved manner with dry-packed cement grout.

#### 5.	The grout shall not be richer than one (1) part cement and three (3) parts sand with the amount of mixing water enough to produce a workable mix.  For exposed walls, the cement shall contain such a proportion of white portland cement as is required to make the color of the patch match the color of the surrounding concrete.  The patch shall be finished in such a manner as to match the adjoining surfaces.

#### 6.	Surfaces of repairs shall receive the same kind and amount of curing treatment as required for the concrete in the repaired section.

### B.	Defective Concrete:

#### 1.	Any concrete which is not formed as shown on the Drawings or does not conform to the Contract tolerances or shows defects which reduce its structural adequacy, shall be removed from the job by the CONTRACTOR at his expense unless the Engineer grants permission to patch the defective area.

### C.	Exposed Concrete Surfaces:

#### 1.	As soon as forms are removed, exposed surfaces shall be carefully examined and all ridges, ribs and other imperfections shall be rubbed with an abrasive stone or ground in a satisfactory manner in order to secure a smooth, uniform and continuous surface.  Plastering or coating of surfaces to be smoothed will not be permitted

#### 2.	No repairs shall be made until after inspection by the ENGINEER.

#### 3.	In no case will extensive patching of honeycombed concrete be permitted

#### 4.	Concrete containing minor voids, pinholes, honeycombing, or similar depression defects shall be repaired as indicated below.

#### 5.	Concrete containing extensive voids, holes, honeycombing, or similar depression defects shall be completely removed and replaced.  Repairs and replacements shall be performed promptly.

## 3-12	REINFORCEMENT:

## A.	Reinforcement shall be in accordance with SECTION 03200 Concrete Reinforcement, of these Specifications.  Concrete protection for the reinforcement shall conform to the requirements ACI 318, paragraph 7.7.1.

## 3-13	CONSTRUCTION TOLERANCES:

### A.	The CONTRACTOR shall set and maintain concrete forms and perform finishing operations to ensure that the completed WORK is within tolerances.  Surface defects and irregularities are defined as finishes and are to be distinguished from tolerances.  Tolerance is the permissible variation from lines, grades, or dimensions indicated on the Drawings.  Where tolerances are not stated in the specifications, permissible deviations will be in accordance with ACI 117.

### B.	The following non-cumulative construction tolerances apply to finished walls and slab unless otherwise indicated:

| Item | Tolerance |
| --- | --- |
| Variation of the constructed linear outline from the established position in plan. | In 10-feet: 1/4-inch; In 20-feet or more: 1/2-inch |
| Variation from the level or from the grades indicated. | In 10-feet: 1/4-inch; In 20-feet or more: 1/2-inch |
| Variation from plumb | In 10-feet: 1/4-inch; In 20-feet or more: 1/2-inch |
| Variation in the thickness of slabs and walls. | Minus 1/4-inch; Plus 1/2-inch |
| Variation in the locations and sizes of slabs and wall openings | Plus or minus 1/4-inch |

3-14	CARE AND REPAIR OF CONCRETE:

### A.	The CONTRACTOR shall protect concrete against injury or damage from excessive heat, lack of moisture, overstress, or any other cause until final acceptance.  Particular care shall be taken to prevent the drying of concrete and to avoid roughening or otherwise damaging the surface.  Any concrete found to be damaged, or which may have been originally defective, or which becomes defective at any time prior to the final acceptance of the completed WORK, or which departs from the established line or grade, or which, for any other reason, does not conform to the requirements of the Contract Documents, shall be satisfactorily repaired or removed and replaced with acceptable concrete.

***END OF SECTION***