SECTION 02820

WIRE FENCES AND GATES

PART 1 - GENERAL

1.01 	SCOPE:

A. 	Summary of Work:  The CONTRACTOR shall furnish all labor, materials and equipment necessary to install woven-wire and/or barbed wire fencing and gates with steel posts as shown on the Drawings and specified herein.

1.02 	APPLICABLE PUBLICATIONS:

A. 	American Society of Testing and Materials (ASTM):

1. 	A47/A47M-99 - Standard Specification for Ferric Malleable Iron Castings.

2. 	A116-00 - Standard Specification for Metallic-Coated, Steel Woven Wire Fence Fabric.

3. 	A121-99 - Standard Specification for Zinc-Coated (Galvanized) Steel Barbed Wire.

B. 	Louisiana Department of Transportation Standard Specifications for Road and Bridge Construction (LDOT).

1.03 	DEFINITIONS:  (Not Used)

1.04 	SUBMITTALS:  Submittals shall be made to ENGINEER in accordance with the following specific information.

A. 	Shop Drawings: Submit shop drawings and installation drawings for the ENGINEER’S review prior to fabrication and delivery.  These drawings shall provide detailed information and specifications for all materials, finishes, dimensions, and erection instructions.

B. 	Like items of material provided hereunder shall be the end products of one manufacturer in order to achieve standardization for appearance, maintenance, and replacement.

1.05 	QUALIFICATIONS:  (Not Used)

1.06 	RESPONSIBILITIES:  (Not Used)

1.07 	CERTIFICATIONS AND TESTING:  The CONTRACTOR shall submit certifications for the fencing materials indicating that they meet the requirements of the specifications.  Certifications required shall include the following.

A. 	Certifications that woven-wire and barbed wire fencing materials meet Specifications.

B. 	Certification that steel galvanized posts conform to Specifications.

1.08 	INSPECTION COORDINATION:  The CONTRACTOR shall provide access to the WORK for the ENGINEER as requested for inspection.  The Contractor shall provide 48 hours notice of its intention to begin new WORK activities.

1.09 	WARRANTY:  (Not Used)

PART 2 - PRODUCTS

2.01 	The CONTRACTOR shall provide materials of manufacturer's standard products where such products conform to Specifications.

A. 	The use of a manufacturer's name and model or catalog number is for the purpose of establishing the standard of quality and general configuration desired only.  Products of other manufacturers will be considered in accordance with the General Conditions and Special Provisions.

B. 	Fencing materials and gates shall be new products of recognized, reputable manufacturers.  All fencing materials and gates shall carry a tag identifying the manufacturer.  Each roll of fencing material shall be legibly marked with the style, class of zinc coating, and other pertinent identifying information.

2.02 	STEEL LINE POSTS:  The CONTRACTOR shall furnish steel post conforming to the following:

A. 	Posts shall be made of 3 inch steel pipe and hot-dipped galvanized after all fabrication and welding is completed positioned as shown on the Drawings

B. 	Minimum length of 7 feet above ground and minimum of 3 feet embedment in ground with a 3 inch concrete casing all around pipe.

C. 	Hot dipped galvanized finished.

2.04 	WIRE FABRIC:  The CONTRACTOR shall furnish wire fabric conforming to the following:

A. 	Conform to ASTM A116, Type I, Woven-Wire, Style 1, No. 11 Farm-Field Fencing.

B. 	47 inches height with 10 horizontal wires, and stays 6 inches on center.

C. 	Top and bottom horizontal wires:  No. 9 gauge.

D. 	Intermediate and stay wires:  No. 11 gauge.

E. 	Zinc-coated, Class 3.

2.05 	BARBED WIRE:  The CONTRACTOR shall furnish barbed wire conforming to the following:

A. 	Conform to ASTM A121-99, 4-point pattern, round barbs spaced five inches on center.

B. 	Strand Size:  No. 12-1/2 gauge.

C. 	Barb Size:  No. 14 gauge.

D. 	Zinc-coated, Class 3.2.06 	GATES AND ACCESSORIES:  The CONTRACTOR shall furnish gates and accessories conforming to the following:

A. 	Gate:  Manual Swing hot dipped galvanized steel gate of standard manufacture in the widths indicated. The gate leafs framing shall be constructed of 2-inch pipe with adequate thickness to support spans shown on Drawings.  Gates posts shall be made of 4 inch steel pipe with a welded base plate and hot-dipped galvanized after all fabrication and welding is completed positioned as shown on the Drawings.  Each gate shall be provided with two latches, two hinges, two five-eighths inch diameter anchor drop bars, link galvanized steel chain, and corrosion resistant padlocks. All gates shall be able to be hinged on either end.

B. 	Hinges: Heavy pattern, malleable, and of adequate strength for gate size. Arranged so gate may be swung back parallel with the barrier.

C. 	Vertical Fence Stays: Vertical fence stays shall be standard gauge, galvanized steel.

D. 	Staples: Formed of No. 9-gauge zinc-coated steel wires one and one-half inch long.

F. 	Bracing Wire:  Bracing wire shall be No. 9-gauge or heavier, zinc-coated soft, smooth wire.

PART 3 - EXECUTION

3.01 	INSTALLATION:  The CONTRACTOR shall install the fence, gates and accessories in accordance with the following:

A. 	Posts: Set posts in 3’ deep hole, plumb.

B.	 Place concrete collar around pipe in hole.’ Replumb.

C.	Post located on drawings; or replace in original locations	for temporarily removes posts and fencing.

3.02 	WOVEN FENCING:

A. 	Attach woven-wire fence to framing pipe with not less than five galvanized ties per each side of steel frame.

B. 	Mount one strand of barbed wire two inches above the woven-wire fencing as shown on Drawing.

3.03 	INSTALLATION OF BARBED WIRE:

A. 	Attach all barbed wire to the framing in accordance with recognized standard practice for first class fence construction and in conformance with these Specifications.

B. 	Terminate the barbed wire at each bottom corner of outrigger framing.  gate post, corner post, and intermediate line post. Each line of barbed wire shall be wrapped around the terminating, corner.

C. 	Splices of barbed wire between frame corners will not be permitted.  In final position, the barbed wire shall be free from sag.

D. 	 Upon completion, the gate shall be straight between corners.  Gate posts shall be vertical and firmly anchored.  All braces, fittings, and fixtures shall be tight and firm, and all barbed wire shall be free from wrap and sag.

3.04 	GATE INSTALLATION:  The CONTRACTOR shall install gates with the faces plumb and the top level.  Install so that the gates will swing freely, without binding, through an arc of 90 - 140 degrees outward.  Gate leafs shall be hung to clear ground to prevent gates from dragging, within the area of the gate swing.

3.05 	CLEANUP:  The CONTRACTOR shall, upon completion of the fence and at such times during the construction as the ENGINEER may direct, clean the fence construction area of trash, debris, and rock.

PART 4- MEASUREMENT AND PAYMENT

4.01	MEASUREMENT AND PAYMENT: All fencing work shall not be measured for payment. CONTRACTOR shall be paid for all fencing work as a lump sum under bid item X-xxx “SECURITY FENCING.”

****END OF SECTION****