SECTION 05310

METAL GRATING

PART 1 - GENERAL

1-01	SUMMARY:

A.	The Work of this Section shall consist of furnishing all the labor, materials, and equipment necessary for installation of heavy duty steel bar grating bridge deck as shown on the Drawings and as specified herein. This section includes:

1.	Prefabricated heavy duty carbon steel bar gratings.

2.	 Prefabricated Support Frames for gratings.

3.	Miscellaneous installation hardware and accessories.

1 -02	REFERENCES:

A.  ASTM A-36 Carbon Steel.

B.  ASTM A-510 Carbon Steel Wire Rods.

C.  ANSI/NAAMM- MBG-532-09 Heavy Duty Metal Bar Grating Manual.

1-03.	SUBMITTALS:

1.	Product Data: The contractor shall submit the manufacturer’s catalog pages

Including load tables, anchor details and standard installation details.

2. 	Shop drawings: The contractor shall submit for approval shop drawings for the fabrication and erection of all gratings based on plan construction drawings. Include plans, elevations, and details of sections and connections as required. Show type, size and location of all fasteners, including the holes in the stringer beam support. All fasteners shall be hot dipped galvanize or stainless steel.

3.	Samples of Grating and Anchorage system shall be submitted for approval

1-04	QUALITY ASSURANCE:

A.	Codes and Standards:  AISI “Specification for the Design of Cold-Formed Steel Structural Members,” AWS “Structural Welding Code,” SDI “Design Manual for Floor Decks and Roof Decks,” comply with applicable provisions except as otherwise indicated.

B	Manufacturer Qualification: A company specializing in the manufacture of heavy metal bar gratings with not less than 5 years of documented experience.

C.	Fabrication tolerances shall be in accordance with applicable provisions and recommendations of ANSI/NAAMM 532-09 Metal Bar Grating Manual.

PART 2 - PRODUCTS

2-01	MATERIALS:

A. Main bearing bars and rectangular cross bars are to be Carbon Steel type ASTM A-36 Carbon Steel. Round Cross bars are to be per ASTM A-510. Steel banding is to be type ASTM A- 36.

B	Sheet Metal Accessories:  ASTM A 526, galvanized.

C.	Galvanizing:  ASTM A 525, G 90.

D.	Galvanizing Repair Paint:  MIL-P-21035 (Ships).

2-02	MANUFACTURERS:

A.	Acceptable Manufacturers include the following listing or approved equal:

1.	Ohio Gratings Inc. 5299 Southway St. SW, Canton Ohio 44706; 800-321-9800

2.	McNicols Co. P O Box 30300 Tampa, FL 33630; 800-237-3820

2-03	MANUFACTURED UNITS:

A. Description: Welded heavy duty steel Grating with hot dip galvanized coating. Fabricated by welding heavy duty cross bars to heavy duty main bearing bars. Both cross and main bars are serrated on top surface.

B.	Include welded anchor blocks ¼” from the bottom surface with hole to accept washer and attachment bolts.

C.	Carbon Steel ASTM A-36 frames (beams, towers and bracing) shall be provided as shown on the contract drawings to support and attach the gratings. Include anchors as shown for anchoring frame tower supports into concrete as shown on the plans. Galvanize frames after fabrication per ASTM A123.

D.	Gratings shall be designed and manufactured to meet the live load conditions of 100 pounds per square foot and a concentrated load of 400 pounds per square foot. Main Bearing bar depth shall be as shown on the contract drawings or as recommended by the manufacturer to meet the loading requirements, and clear span conditions.

E.	Fabrication tolerances shall be in accordance with ANSI/NAAMM MBG 532-09 Metal Bar Grating Manual.

F.	Gratings and frames shall be Hot-Dip Galvanized per ASTM A123 after all fabrication, including drilling bolting holes.

G.	Provide appropriate fasteners for type, grade, and class required for the approved anchorage system. Include lifting devices if shown on the drawings.

PART 3 - EXECUTION

3-01	INSTALLATION:

A.	Install grating in accordance with shop drawings and standard installation clearances as recommended by ANSI/NAAMM MBG-532-09 Metal Bar Grating Manual.

B.	Use approved attachment system and fasteners to secure grating to supporting members as shown on plans.

3-02	FIELD PROTECTIVE COATINGS:

A.	Surface Preparation:  If grease or oils are present, SP1 - Solvent Cleaning must precede any other method specified.  Prepare all surfaces by SSPC-SP11 and 1 mil profile depth.

1.	Clean all shop coated surfaces damaged from rust and mill scale, welding and abrasion.

B.	Field spotting coat:

1.	Apply to cover all blemished parts of the steel galvanized coating furnished under this Contract.

2.	For hot dipped galvanized surfaces scratched or cut edge, apply 4 mills of Rust-oleum High Performance Galvanizing Compound No. 3BU16, 95%. pure zinc, or approved equal.

PART 4 - MEASUREMENT AND PAYMENT

4-01	The measurement for payment for the heavy duty steel gratings will be based upon the square feet of each panel installed over the steel bridge framing. Price and payment shall constitute full compensation for furnishing all plant, labor, materials, coating splicing, fasteners, consumables and equipment for performing satisfactory installation of grating work. Payment for heavy duty steel grating shall be made under bid item 1” OPEN GRID SERRATED GLAV. DECK PANELS: based on unit price per square foot grating times the accepted square feet installed.

***END OF SECTION***